# ICT_G11_U05_S11 — Interactive Tools

Grade 11 ICT (Ethiopian curriculum). Textbook sub-chapter: 5.1 Hardware Troubleshooting.
Structure: 1. Welcome & Introduction to the section • 2. Core concept teaching & examples • 3. Summary & section conclusion.

# MUST-COVER CHECKLIST (teach every item)

• Hardware Troubleshooting Procedures

• Check POST

• Beep Codes

• BIOS Information

• CMOS Error

• Event Viewer

• Application

• System

• Security

• Hardware Problems

• Diagnostic Tools

# LMS LESSON (authoritative)

PARAGRAPHS

Grade 11 · ICT · practise the key ideas of this unit.

Flip through the key terms and definitions.

Sort concepts into the correct categories.

Test yourself with multiple-choice questions.

# TEXTBOOK CONTENT (authoritative)

5.1. Hardware Troubleshooting

Brainstorming 5.1

 What do you know about computer hardware troubleshooting and

maintenance?

Hardware troubleshooting is a systematic approach to locating the cause of a fault

in a computer system and solving technical problems. It starts with general issues

and then gets more specific.

5

UNIT HARDWARE

TROUBLESHOOTING

AND PREVENTIVE

MAINTENANCE

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 110

5.1.1 Hardware Troubleshooting Procedures

Hardware troubleshooting is the process of reviewing, diagnosing, and identifying

operational or technical problems within a hardware device or equipment. It aims

to resolve physical and/or logical problems and issues within computing hardware.

On the other hand, software troubleshooting is the process of scanning, identifying,

diagnosing, and resolving problems, errors, and bugs in software.

Computers can malfunction and get damaged if computer users are not aware of

some of the basic procedures for checking hardware problems. Many computer

problems can be solved by checking the following simple hardware problems:

• Check that your computer is plugged into a working power outlet.

• Check that everything is turned on.

• If the computer is on but the screen is blank, there may be an issue with the

connection between the computer and the screen. First, check to see if the

monitor is plugged into a power outlet and if the connection between the

monitor and computer system unit is connected securely.

• Check that the keyboard, mouse, monitor, speakers, etc. are properly plugged

into the computer system. Try a different port to check if it is a port issue, or

change the device if the device is damaged.

Notes

 It is necessary to switch off the computer before undertaking any

hardware maintenance such as removing or replacing computer parts.

5.1.2 Check POST

POST stands for Power On Self-Test. This is part of a computer’s startup program

that is used to diagnose the keyboard, the Random Access Memory (RAM), disk

drives, and other hardware to make sure they are working properly. If the POST

detects any errors in the hardware, it either displays a text error message on the

screen or emits a series of short and long beeps.

If an error message appears as you boot your computer, type the exact error message

and then search on the Internet to find more information about the error.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 111

Activity 5.1

1. If your computer cannot start, what are the preliminary hardware diag-

noses you do to identify and fix the problems?

5.1.3 Beep Codes

Beep codes are sounds emitted by the computer during Power on Self-Test (POST).

Each BIOS manufacturer has a unique beep sequence, a combination of long and

short beeps, for hardware failures. If there is a problem with the computer, listen

for the beep codes when the computer starts. As the system proceeds through the

Power on Self-Test (POST), most computers emit one beep to indicate that the

system is booting properly. If there is an error, you might hear multiple beeps. You

need to document the beep code sequence and search on the Internet to determine

the specific problem. Some of the beep codes and the respective problems are as

follows:

• No beep but the system turns on and runs fine - Under normal

circumstances, most computer systems will beep one short beep when turned

on. If your computer doesn’t produce a beep sound, your “beeper” may have

died out.

• No beep - The power supply is not plugged in or turned on. If not, the power

supply is completely dead.

• Steady, short beeps - The power supply may be bad or the voltages might

be wrong. A replacement would usually be necessary.

• Steady, long beeps - The power supply has gone bad.

• Long, continuous beep - Your Random Access Memory (RAM) sticks may

have gone bad. If there is more than one stick installed, try taking one out

to see if the computer boots. If it does not, try the same thing with the other

stick. This will tell you which stick has gone bad, and you can replace or

upgrade accordingly. If there is only one stick installed, you will need to

replace or upgrade it to fix the problem.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 112

• One long, two short beeps - There has been a video card failure. Your first

action is to try reseating the video card. This often solves the problem when

the computer system is connected to projectors because the VGA/DVI or

Video cable gets moved so often that the card can be slowly unplugged. If

reseating doesn’t work, replace the video card.

5.1.4 BIOS Information

BIOS stands for basic input/output system. BIOS is a program used by a computer

to start the computer system after it is powered on. It also manages data flow

between the computer’s operating system (OS) and attached devices, such as the

hard disk, video adapter, keyboard, mouse, and printer. If the computer boots and

stops after the POST, your computer has a BIOS setting problem. Fixing BIOS

problems requires a good knowledge of computer hardware. Therefore, when you

face a BIOS setting problem, you are advised to contact a computer hardware

technician to solve the problem.

5.1.5 CMOS Error

The CMOS (Complementary Metal-Oxide Semiconductor) is an onboard chip that

stores information ranging from the time and date to system hardware settings;

its primary function is to handle and store the BIOS configuration settings. If a

computer shows a CMOS alert message on the screen, it indicates that the CMOS

battery needs to be replaced. Upon receiving such type of error message, remove

the CMOS battery carefully, and insert a new battery that is exactly the same as the

old one.

Notes

 Replacing a CMOS battery may be more difficult in laptop computers

than in desktop computers. If the user of the computer does not have

sufficient computer hardware troubleshooting experience, leaving the

task to a professional computer technician is advised.

5.1.6 Event Viewer

When system or application errors occur on a computer running Windows, the

Event Viewer is updated with information about the errors. The Event Viewer records the following information about the problem:

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 113

• The problem that occurred

• The date and time of the problem

• The severity of the problem

• The source of the problem

• The event ID number

• Which user was logged in when the problem occurred

The following steps can be followed to launch the Event Viewer:

1. On the Windows Search box, write event viewer

2. A pop-up menu appears which looks like the one

3. Click on Event Viewer

Figure. 5.1 Launching Event Viewer

Events are placed in different categories as shown on the left side of Figure 5.2.

Expand each category to get more information. Each category is related to a log

that Windows keeps on events regarding that particular category. While there are

a lot of categories, the vast amount of troubleshooting you might want to do is

related to the Windows Log category, which contains the following items:

• Application: The Application log records events related to Windows system

components, such as drivers and built-in interface elements.

• System: The System log records events related to programs installed on the

system.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 114

• Security: When security logging is enabled (it is off by default in Windows),

this log records events related to security, such as logon attempts and resource

access.

When you click on Application under Windows log, you get a list of Application

log records events.

If you want to get detailed information about the error, double-click on the error,

and then you get detailed information in the pop-up window. Although the Event

Viewer lists details about an error, you might need to do further searching on the

Internet about the problem to identify an appropriate solution.

5.1.7 Hardware Problems

Many computer problems are caused by hardware failures or problems with

hardware drivers. Windows usually displays notifications about devices that have a

problem. Device Manager is used to check the status of different hardware devices.

The following steps can be followed to identify hardware problems in Windows-

based systems:

1. Click on the Windows search box in the lower-left corner.

2. Type Control Panel.

3. Double-click the Control Panel on the Windows pop-up menu.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 115

4. Click Hardware and Sound.

5. Under Device and Printers, Click on Device Manager

The Device Manager has the following four benefits.

1. It works as a centralized utility from which all the hardware on a system

can be configured.

2. It provides a central and organized view of all hardware- Microsoft

Windows-recognized hardware- installed on a system.

3. It helps to manage all the hardware devices installed on a system. This

includes keyboards, hard disk drives, USB devices, etc.

4. It helps to change hardware configuration options, manage drivers, enable

or disable hardware, identify conflicts between hardware devices, etc.

When you click on the Device Manager on the Control Panel the Device Manager window is displayed. The devices that

have a problem would have an error icon displayed right next to the name of the

device. The operating system flags the devices with an error icon.

• A yellow triangle with an exclamation mark indicates that the device has

a problem.

• A red X means that the device is disabled or removed or Windows can’t

locate the device.

• A downward-pointing arrow means the device has been disabled.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 116

• A yellow question mark indicates that the system does not know which

driver to install for the hardware. This problem will be solved by installing

the appropriate driver software for the device

Activity 5.2

1. Open the Device Manager and check if there is any problem on your

DVD/CD-ROM drives.

2. If you find a yellow triangle with an exclamation mark on one of your

hardware devices, what will you do to solve the problem?

5.1.8 Diagnostic Tools

Diagnostic Tools are software tools that are used to help troubleshoot, diagnose

and solve hardware problems. Manufacturers of system hardware usually provide

diagnostic tools of their own. For instance, a hard drive manufacturer might provide

a tool to boot the computer and diagnose why the hard drive does not start the

operating system.

The top two diagnostic tools are Windows Performance Monitor and Windows

Resource Monitor.

a) Windows Performance Monitor

The performance monitor gives a quick view of vital information about computer

hardware. The computer’s CPU, Memory, Disk, and Ethernet information can be

checked from there. Performance Monitor is used to examine the effects of running

applications in both real-time and by collecting data to check out for later analysis.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 117

For example, to view the Performance Monitor, the following steps can be followed:

1. Press CTRL + ALT + Delete button at the same time.

2. Choose Task Manager, and the window shows that appears in Figure 5.5.r

3. Then click on the Performance tab to see the performance of the CPU and

other devices in the computer.

Notes

 One quick way of reducing the load from the CPU in Windows is

to restart the computer to remove any unwanted temporary files. Make

sure that all files are saved before proceeding with this step.

 The other option is to look for the applications that are using maximum

CPU resources on the Task Manager. If any application shows CPU

usage of almost 100%, disable the application and then start it again.

Activity 5.3

• Open Task Manager and see if any applications are using excessively

large amounts of the computer’s CPU or memory, and if there are any,

disable them.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 118

b) Windows Resource Monitor

Windows Resource Monitor is better suited for tracking CPU, Network, Memory,

and Disk usage. This tool allows to take an in-depth look into which processes are

affecting the CPU, how much memory is being used, the disk activities, and the

network information such as current TCP (Transport Control Protocol) connections,

and which processes are listening on which port.

The following steps can be followed to open the Windows Resource Monitor:

1. On the Windows search box, write Resource Monitor

2. Click on the Resource Monitor, and then the window shows what appears

in Figure 5.6.

Clicking on the CPU tab in the Windows Resource Monitor lists the four sections

namely, Processes, Services, Associated Handles, and Associated Modules. The

processes that are running are shown in black color under the Processes section,

and those that are suspended are shown in blue color while the processes that are

not responding are shown in red color. Upon selection of a specific running process

from the Processes section the related data under the Services, Associated Handles,

and Associated Modules get populated.

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 119

If you find your computer slowing down unexpectedly, take a look at the CPU

column. If an application is taking up a lot of CPU resources, shut down the

application and restart it.

To stop the application

1. Open resource monitor window

2. Right-click on the application

3. Click on End process

If you want to know more about an application, you can follow the following steps

in the Resource Monitor window:

1. Right-click on the name of the application

2. Choose Search online

This opens your default browser displaying the search result of the application on

the default search engine of your browser. Click on the application links and learn

more about the application.

Activity 5.4

1. What do you understand by hardware troubleshooting?

2. If your computer is slow, what will be the possible problem and what

course of action can you take to solve it?

I NFORMATION TECHNOLOGY GRADE 11 ~ STUDENT TEXTBOOK 120